EE 272L: Digital Systems

2nd Year, Spring Semester 2026

Laboratory Instructor

Ms. Mahnoor Farhat

Student Name:                               Muhammad Zain

Registration Number:                       2024-EE-081

Section:                                                         B

Department of Electrical Engineering (DEE),

University of Engineering and Technology (UET),

Lahore

Answers to the given question in section 3.2.1:

1. What are the names of the input and output ports of both the decoders?

Decoder 1:

It has two inputs and one output. Inputs are sum[2:0] and c_out. And output is seg[6:0]. The seg port drives the segment of the display (CA,CB, CC, CD, CE, CF, CG).

Decoder 2:

It has one input and one output. Input is sel[2:0] which selects only one of the available seven segment displays. And output is an[7:0] which enables one seven segment display to function.

2. What is the size of all the ports of both the decoders?

Decoder 1: Takes 2 inputs sum[2:0] of size 3 bits and c_out of size 1 bit, collectively input size is 4 bits and output (seg[6:0]) size is 7 bits.

Decoder 2: Takes one input sel[2:0] of size 3bit. And output (an[7:0]) size is 8 bits.

3. How many columns will be there for both the truth tables? Justify.

Decoder 1: 11 columns, 4 columns for 4 input bits + 7 columns for 7 output bits.

Decoder 2: 11 columns, 3 columns for 3 input bits + 8 columns for 8 output bits.

4. How many rows will be there for both the tables? Justify.

Decoder 1: 16 rows. 4-bit input gives 2^4 = 16 combinations.

Decoder 2: 8 rows. A 3-bit input gives 2^3 = 8 combinations.

5. Map the MSB of seg to the CA port and the MSB of the an to AN7 port of the seven segment display. How will the rest of bits of seg and an ports be mapped to the inputs of the seven segment display i.e. which output port of the decoder gets connected to which input port of the seven segment display?

Segment Mapping:

| seg[0] | CG |
| --- | --- |
| seg[1] | CF |
| seg[2] | CE |
| seg[3] | CD |
| seg[4] | CC |
| seg[5] | CB |
| seg[6] | CA |

Anode Mapping:

| an[0] | AN0 |
| --- | --- |
| an[1] | AN1 |
| an[2] | AN2 |
| an[3] | AN3 |
| an[4] | AN4 |
| an[5] | AN5 |
| an[6] | AN6 |
| an[7] | AN7 |

6. How many Boolean expressions are needed?

A total of 15 Boolean expressions are needed, 7 for Decoder 1 and 8 for Decoder 2.

Decoder 1

Truth Table:

|  |  |  |  | Component Names | CA | CB | CC | CD | CE | CF | CG |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | FPGA Pin Names | T10 | R10 | K16 | K13 | P15 | T11 | L18 |
| c_out | sum[2] | sum[1] | sum[0] | Port Names | seg[6] | seg[5] | seg[4] | seg[3] | seg[2] | seg[1] | seg[0] |
| 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 0 | 0 | 0 | 1 |  | 1 | 0 | 0 | 1 | 1 | 1 | 1 |
| 0 | 0 | 1 | 0 |  | 0 | 0 | 1 | 0 | 0 | 1 | 0 |
| 0 | 0 | 1 | 1 |  | 0 | 0 | 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 0 |  | 1 | 0 | 0 | 1 | 1 | 0 | 0 |
| 0 | 1 | 0 | 1 |  | 0 | 1 | 0 | 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 0 |  | 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 1 |  | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 |  | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| 1 | 0 | 1 | 0 |  | 0 | 0 | 0 | 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 1 |  | 1 | 1 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 0 | 0 |  | 0 | 1 | 1 | 0 | 0 | 0 | 1 |
| 1 | 1 | 0 | 1 |  | 1 | 0 | 0 | 0 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 |  | 0 | 1 | 1 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 |  | 0 | 1 | 1 | 1 | 0 | 0 | 0 |

Boolean Expressions: (SOP)

CA = (S̄3. S̄2 . S̄1 . S0) + (S̄3. S2 . S̄1 . S̄0) + (S3. S̄2 . S1 . S0) + (S3. S2 . S̄1 . S0)

CB = (S̄3. S2 . S̄1 . S0) + (S̄3. S2 . S1 . S̄0) + (S3. S̄2 . S1 . S0) + (S3. S2 . S̄1 . S̄0) + (S3. S2 . S1 . S̄0)  + 
          (S3. S2 . S1 . S0)

CC = (S̄3. S̄2 . S1 . S̄0) + (S3. S2 . S̄1 . S̄0) + (S3. S2 . S1 . S̄0) + (S3. S2 . S1 . S0)

CD = (S̄3. S̄2 . S̄1 . S0) + (S̄3. S2 . S̄1 . S̄0) + (S̄3. S2 . S1 . S0) + (S3. S̄2 . S1 . S̄0) + (S3. S2 . S1 . S0)

CE = (S̄3. S̄2 . S̄1 . S0) + (S̄3. S̄2 . S1 . S0) + (S̄3. S2 . S̄1 . S̄0) + (S̄3. S2 . S̄1 . S0) + (S̄3. S2 . S1 . S0) +

(S3. S̄2 . S̄1 . S0)

CF = (S̄3. S̄2 . S̄1 . S0) + (S̄3. S̄2 . S1 . S̄0) + (S̄3. S̄2 . S1 . S0) + (S̄3. S2 . S1 . S0) + (S3. S2 . S̄1 . S0)

CG = (S̄3. S̄2 . S̄1 . S̄0) + (S̄3. S̄2 . S̄1 . S0) + (S̄3. S2 . S1 . S0) + (S3. S2 . S̄1 . S̄0)

Decoder 2

Truth Table:

| Component Name | SW15 | SW14 | SW13 | AN7 | AN6 | AN5 | AN4 | AN3 | AN2 | AN1 | AN0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FPGA Pin Name | V10 | U11 | U12 | U13 | K2 | T14 | P14 | J14 | T9 | J18 | J17 |
| Port Name | sel[2] | sel[1] | sel[0] | an[7] | an[6] | an[5] | an[4] | an[3] | an[2] | an[1] | an[0] |
|  | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |
|  | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 |
|  | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 |
|  | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 |
|  | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 |
|  | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 |
|  | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 |
|  | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Boolean Expressions: (POS)

AN7 = X̅2 + X̅1 + X̅0

AN6 = X̅2 + X̅1 + X0

AN5 = X̅2 + X1 + X̅0

AN4 = X̅2 + X1 + X0

AN3 = X2 + X̅1 + X̅0

AN2 = X2 + X̅1 + X0

AN1 = X2 + X1 + X̅0

AN0 = X2 + X1 + X0